Addendum to 2006 F3D rule proposals

F3D 2006 statistics of positions  of ground impact from crashes and mid-air collisions.

The crashes during a number of 2006 F3D competitions are summarised.

Some facts and figures:

- 6 competitions: Euro-cups in Siziano, Melnik, Tours and Quedlinburg, Deutsche Meisterschaft in Cloppenburg and Riepe,  Netherlands championship in de Peel

- Total approximately 250 entries, approximately 1500 competition flights and 800 practice flights

- recorded 24 crashes, position of ground impact shown in diagram below.

Conclusions and recommendations:

- The great majority (20 out of a total of 24) of ground impact is in a zone of approximately  30 metres outside the lines that make the triangle of the nominal race course, no impact inside the triangle. This supports the idea that the 20 metre circle is a safe area. These type of crashes can be considered “safe”

- most ground impact is outside the pylons 1 and 3, mostly due to piloting errors and “bad air” from other planes in the turns. These type of crashes can be considered “safe”

- many crashes are on the straight between pylon 3 and pylon 1, just by flying too low. (This type of crashes can be considered “safe”)

- The ground impact from mid air collisions is more frequently (2 out of 4)  further away from the nominal race course and are less predictable and therefore potentially more dangerous. However, in these cases the speed of the ground impact is reduced from losing a wing or the fact that the pilot has shut the engine immediately.

- The safety area defined as the line outside the line whereon the sideline judges are,  appears to be reasonably adequate, although an increase of this distance may be considered if the site allows it. Probably the statistically most dangerous positions on this line are between the pylon 2 and pylon 3 judges. ( The Ready Area, contest administration and  fuelling station are often located here and the position of these should be reconsidered.) A net over an appropriate length, approximately 5 meters, in front of the safety line could also be considered for further protection.

- Although the number of observations is very limited,  it looks more safe and should therefore be preferred to have the judges and the pits on the other side of the race course than the drawing above and position No. 1 pylon on the left side seen from the pits and Nos 2 & 3 pylons at the right side. But the wind direction and the basic lay-out of the airfield may make such an arrangement not possible in a number of cases.

- At this moment there are rules concerning low flying. Low flying aircraft will however crash relatively close to the course; an aircraft that hits a pylon will never fly more than 50 metres, continuing more or less its original flight direction. More attention should be given to high and wide flying aircraft, because these will crash outside the safety zones (e.g. by a radio failure). This may be part of the side line judges duty, where of course workable definitions will have to be developed for a dangerous flying style.

- The monitoring of ground impact should be reported to the F3D Sub-committee by the organisers as a standard routine to continue this type of statistics in order to work on a maximally safe competition site lay – out.